Forecasting Ash Concentrations in Volcanic Clouds Using Dispersion Models

Part I:  Sources and Significance of Uncertainty in Model Forecasts

Contributors:  Costanza Bonadonna, Roger Denlinger, Arnau Folch, Marianne Guffanti, Matthew Hort, Ian Lisk, Sue Loughlin, Larry Mastin, Rene Servranckx, Barbara Stunder, Andrew Tupper, Claire Witham.

Introduction

Numerical models of volcanic ash transport and dispersion are used to forecast the location and concentration of volcanic ash plumes in a quantitative manner that is not possible through most remote sensing or other observational means.  The 2010 eruption of Eyjafjallajökull in Iceland prompted a shift in risk-management procedures in Europe to allow flight through zones of dilute ash under some circumstances.  As evidenced by the work plan of the International Civil Aviation Organization (ICAO) International Volcanic Ash Task Force (http://www2.icao.int/en/anb/met-aim/met/ivatf/Lists/Meetings/AllItems.aspx/), there is now an enhanced interest in the ability to forecast volcanic ash concentration using dispersion models.

A workshop on ash dispersal forecasting and aviation was held in Geneva, Switzerland, on 18-20 October 2010, under the auspices of the WMO, International Association of Volcanology and Chemistry of the Earth’s Interior, and University of Geneva.  The workshop was a forum for scientists to document the capabilities and sensitivities of different dispersion models and ash-detection techniques and to identify possible improvements related to input parameters, data assimilation, probabilistic forecasts, and ensembles.  Technical results from the workshop are documented and available at www.unige.ch/hazards/Workshop.html.

In this paper, we examine an aspect of modeling pertinent to the operational (decision-making) use of dispersion forecasts—namely, the sources and significance of the uncertainties inherent in such forecasts.

First, some definitions:  A forecast of the expected position and concentrations of an ash plume in space and time is a solution of the governing equations of a particular dispersion model for a given set of inputs. The uncertainty of a forecast can have two origins:  model inaccuracies and input uncertainties.  Accuracy is an intrinsic property of a model and refers to the capacity of the model to be correct with respect to actual conditions.  Model accuracy is dependant on the underpinning model science and crucially on the input parameters used to initialize the model.  Input uncertainty reflects our incapacity to furnish precise model inputs for whichever reason, e.g. uncertainties in input detection, uncertainties related to the variability of the natural system.  A very accurate model can produce highly uncertain forecasts if model inputs are not constrained.

Dispersion model accuracy is related to the integrity of the governing equations and constitutive laws (e.g., particle aggregation, deposition mechanisms) and the integrity of the various numerical techniques (e.g., solution of diffusion equation).  Input uncertainty mostly derives from estimates of parameters at the eruptive sources (e.g., plume height, mass of material erupted, particle properties) and the quality of the meteorological fields (variables like wind field or temperature are, in turn, driven by numerical weather prediction (NWP) models and therefore have also a degree of uncertainty) and the variability of the natural system.

In the next section, we describe sources of forecast uncertainty in terms of three components comprising input uncertainty and model inaccuracy:  (1) estimates of eruption source parameters, (2) downwind ash removal processes and atmospheric transport models, (3) meteorology/numerical weather prediction.  In later sections, we highlight ways to decrease forecast uncertainty and some of the implications of forecast uncertainty for operational products.

Sources of Forecast Uncertainty

(1) Eruption Source Parameters (ESP)

These parameters comprise all non-meteorological parameters that characterize an eruption and the input of volcanic ash to the atmosphere at the volcanic source.  ESP vary among volcanoes, as well as during multiple eruptive events at a particular volcano.  ESP primarily consist of:

- Start and end time of eruption:  These parameters are fundamental input parameters for all dispersion models and can be difficult to obtain accurately if the eruption is obscured or the volcano is not adequately monitored.

- Height of the eruptive plume:  This parameter determines what wind fields and weather systems transport the ash mass.  Depending on the magnitude of explosive eruptive activity, plume heights can range from less than a kilometer to nearly 50 km, and plume height can vary over time as a particular eruption progresses.  Plume height can be estimated using several satellite techniques, with ground-based radar, or visually by ground observers or from aircraft reports.   All these types of estimates have uncertainties and present a considerable challenge when meteorological clouds are present or for remote/unmonitored volcanoes.  In the case of the Eyja eruption, plume height estimation was carried out by the responsible State Volcano Observatory, the Iceland Meteorological Office (IMO), primarily using a Doppler weather radar.  It was estimated that for this event maximum plume height errors were generally no greater than approximately one kilometer although the pulsing nature of this long lasting eruption did add another layer of complexity to these estimations. Therefore it is recommended that:  i) multiple plume height assessments are provided derived from different techniques, as each technique is associated with intrinsic limitations and uncertainties; (ii) the part of the plume/cloud for which the height is derived needs to be specified (e.g., neutral buoyancy level, overshooting, top of umbrella cloud); (iii) the horizontal distance from the vent at which the height is detected also needs to be specified (in particular for bent-over plumes); (iv) a better standardization among different communities is required for the determination of plume height (e.g., height should always be reported above sea level and consistently relative to the same datum).

- Mass eruption rate (MER):  This is the rate (mass per time) at which ash is emitted into the atmosphere.  Ash concentration in volcanic clouds is directly related to this rate, which ranges over more than five orders of magnitude for historical eruptions worldwide.  MER is hard to measure directly and a distinction should be made between MER (i.e., at vent), mass transport rate (MTR) in the cloud at the neutral buoyancy level and local mass transport rate (i.e., at a given distance from the vent). A distinction should also be made between MER/MTR of all particle sizes and MER/MTR of small particles (i.e., particles detected by satellite sensors). Ash dispersal forecasting associated with aviation safety and long-range dispersion mainly requires information on MER of fine particles that enter the horizontally-spreading cloud. If MER is calculated from plume column height, then the most appropriate parameterization should be used based on plume dynamics (e.g., thermal vs. sustained plume, strong vs. weak plume).  Examples of techniques that could help constrain MER/MTR (of selective particle sizes) are radar, LIDAR, ground-based IR or UV camera, satellite, seismic energy release, infrasound, and airborne monitoring for local MER.  There is considerable scatter in the empirical relationship between MER and column height, which reflects both real variance and measurement errors.  A column height of 10 km correlates best with an eruption rate of about 1.8 million kg/s; but within one standard-deviation error, it could range from approximately 0.7 million kg/s to approximately 8 million kg/s — more than an order of magnitude.  The MER can vary quickly with changes in eruptive vigor during an eruption and is much more difficult to determine for a weak plume than a strong one.

- Mass distribution of material in the plume by elevation:  Volcanic plumes are driven upward by buoyancy of hot gas and air.  Large eruptions produce so much heat that ash columns can ascend over 100 km per hour to an elevation that significantly overshoots the altitude where the plume’s density becomes equal to the surrounding atmosphere.   These rapidly rising columns typical of large eruptions are unlikely to be bent over by wind, thus forming a “strong” plume that spreads laterally, due to gravitational spreading, near its top to form a stratospheric umbrella cloud, with most mass is concentrated in the umbrella top.  In contrast, small eruptions rise slowly and are easily affected by wind to form a bent or “weak” plume.  Weak plumes distribute mass more evenly over a wider range of elevation in the atmosphere.  Dynamics of strong and weak plumes are very different. As a result, appropriate parameterization is required.

- Particle-size distribution:  This parameter affects how long ash particles stay aloft to be dispersed and has direct influence on ash concentrations.  Gravitational force causes erupted fragments (called tephra) to fall out from the atmosphere and eventually settle to the ground, with the rate of fallout depending on particle size and density.  Ash is tephra particles less than 2 mm (2000 microns) in diameter.  Fragments larger than one hundred microns can fall at a meter per second or faster, reaching the ground within several hours and usually within a few hundred kilometers of the volcano.  Micron-sized fragments theoretically fall at centimeters per second or less, staying in the atmosphere for days.  Particle-size distribution varies with eruption size, style, and magma composition but is not well constrained because most of our knowledge comes from deposits that fall from the ash cloud—not the cloud itself.

(2) Downwind ash-removal processes and atmospheric transport models

In addition to gravitational fall-out there are various processes that occur during atmospheric transport which cause ash to be removed from the atmosphere.  Many of these processes will change during the course of an eruption and are inherently difficult to model but can be approximated using theoretical or semi-empirical relationships.

- Ground deposits of tephra clearly show that fine particles can clump together (aggregate) to form larger clusters with larger diameter and faster fallout rates.  It is thought that aggregation occurs by electrostatic attraction, the incorporation of water at the eruptive source (magmatic water, ice cap water, ground water) and mechanical collision in the atmosphere

- Encasement of ash particles in ice crystals will increase the fallout rate of fine particles.

- Precipitation processes such as scavenging by raindrops and raindrop formation also remove ash from volcanic clouds.

The representation of these processes varies considerably among models, both NWP and ash dispersion models.  Also data for verification is very poor.  Most dispersion field experiments are designed to avoid rain, so data on scavenging coefficients etc are poorly known. For deposition, dispersion models depend on what is extracted from the NWP models, but cloud and precipitation are not the most accurate (paired in space and time) output quantities from NWP models.

Several different atmospheric transport models are used by the VAACs for operational response and by various other groups for research and development purposes.  There are valid scientific reasons for having different dispersion models, e.g., stimulating scientific development, exploration of the suitability and sensitivity of modeling techniques.  For full documentation of model differences, see the summary from the Geneva workshop of the main characteristics of 12 volcanic ash transport and dispersal models (Bonadonna et al. 2011b) (http://www.unige.ch/sciences/terre/mineral/CERG/Workshop/results/Model-Document-Geneva10.pdf/).    There are some important points to keep in mind with respect to model comparisons:

- In addition to using different modeled physics and numerical techniques, models can differ in terms of procedural/setup differences.  Many of the differences that are perceived to be significant among models vanish once the models are actually set up in a comparable way.  It is far more important to consider best practice in model set up and observational data inputs than direct model differences.  From a dispersion perspective, this is not just a volcanic ash issue; these points come up in the wider literature for almost all dispersion model applications.

- Model comparisons need to be set up very carefully and strong conclusions can’t be drawn for only a few comparisons.  The effort required to quantify the issues and identify the most important ones is considerable and has only begun.

- While some models may have much functionality, the science behind these features might not be as robust as that found in other models which apparently have less functionality.  It is a challenge to make sound judgments about what is the best science in many of the areas of relevance to atmospheric transport models.  The science is not yet extensively supported in the existing literature, and for many of the features/parameterizations that are important the literature is incomplete or varied. It must also be remembered that there are invariably many ways or combination of model capabilities that might enable  a suitable level of accuracy to be achieved.

(3) Meteorology / Numerical Weather Prediction (NWP)

Atmospheric NWP models can run at global, regional and local scales, and their accuracy is affected by their underlying science and the quality and quantity of supporting observational data:

- The predictive skill of modern NWP models is generally high. Significant use of remote sensing data in data assimilation routines means that this remains true even in traditional remote areas such as over the oceans or in the upper atmosphere.  Nevertheless, the uncertainties vary from forecast to forecast, from one location to another and over the forecast duration even from the same model, let alone between models.  There is an ongoing quality control and model improvement cycle at all National Meteorological Centers that host the VAACs, although their measures are focused on the traditional roles of NWP and may not be the best measures for dispersion model predictions.

- For volcanic-ash forecasting, upper level winds are typically more important than the surface winds.  In NWP, limited area models, typically used at higher resolution to give better forecasts, often are considered better and therefore used by default in preference to the coarser global configurations.  However, due to boundary condition issues limited area models can be less accurate at upper levels than global model configurations.

- Even small timing/positional errors in e.g., passage of an upper trough within an NWP forecast can, if at an unfortunate location/time, have a significant impact upon the dispersion prediction.  This arises because the volcanic plume is being released at a very specific time and place, so exact timing and positional relationships become critical for the dispersion prediction even if they appear small from a weather prediction point of view.

- In NWP models, the details of the earth's topography are constrained by the spacing of the horizontal and vertical grids. A very high, steep mountain is represented as a smoothed, rounder and flatter surface in models.  A somewhat extreme example of this: the topography of an operational NWP model with a horizontal grid spacing of 15 km / 80 vertical levels used by a specific VAAC has its highest point at 3523 meters in the vicinity of Mount McKinley, Alaska.  In reality, the summit of Mount McKinley is at 6194 meters.  This is a problem for all NWP models and VAACs, whenever the topography varies rapidly in space. It also highlights a danger with trying to predict ash concentrations for small atmospheric layers in regions of rapidly varying topography. By the very nature of volcanoes, this is almost always an issue in regions where they are located.

Summary of Sources of Uncertainty

It is difficult to quantify the combined impact of the uncertainties discussed above, but we can say that the mass eruption rate represents the biggest source of input uncertainty, particularly for ‘remote’ volcanoes, whilst the imperfect modeling of ash removal processes represents another significant source of error.

Clearly, there is a need for more extensive model sensitivity studies.  From a scientific point of view, a specific parameter may seem important, but in practice if it only changes the results by 10%, it should not be near the top of any wish list to resolve from an operational perspective. Responses and sensitivities may also vary significantly between modeling approaches, differing eruptions and differing observational inputs.

Approaches for Decreasing Uncertainties in Model Forecasts

Uncertainty in modeled volcanic ash forecasts can be decreased on two fronts: (1) increasing accuracy and (2) decreasing input uncertainties.

Model forecast accuracy can increase by better representation of critical ash deposition mechanisms (primarily aggregation and wet deposition) and other improvements to model physics.

Ways of reducing the uncertainties associated with ESP (plume height, grain-size distribution, MER and vertical mass distribution) include using more accurate values through assimilation of observations of ash plume features into models and characterization of the uncertainties that are inevitable in any natural system.  The accuracy of the ESP can increase with time as data (first from observations at the vent and later from, airborne samples, remote-sensing retrievals, etc.) are obtained.  For this reason it is important to distinguish between pre-eruption forecasting, first simulation, and during-eruption forecasting.

- Pre-eruption forecasting (i.e. assuming an eruption scenario) should be based on dedicated ESP probability density functions (PDFs) compiled for each given volcano or, in cases where  dedicated PDFs are not available, on standard ESP that account for related uncertainties (e.g. Mastin et al. 2009). The use of PDFs implicitly implies model ensemble on input conditions but not necessarily a probabilistic output.

- The first simulation is run just after the onset of an eruption and can rely on ESP that can be detected in close to real-time (i.e. plume height). In fact, plume height which is of fundamental importance for the initialization of all dispersion models, is usually the easiest parameter to constrain in close to real-time (e.g. using radar, satellite, PiReps or ground visual observation, thermal camera). Later on during the eruption, plume-height determination can be further supplemented by the following capabilities if available - research aircraft measurements, Unmanned Airborne Vehicles (UAV) and aerosol sondes. Other crucial ESP can be derived from plume height (e.g., mass eruption rate and erupted mass) using appropriate empirical equations, whereas grain-size distribution will not be available at the time of the first simulation and, therefore, will still have to rely on PDFs or standard values with the limitations discussed above.

- During-eruption forecasting can benefit from data assimilation of ESP of increasing accuracy as soon as these are made available.  Observations should be assimilated into models from both direct measurements (e.g. source terms/characteristics) and indirectly through combination with other models (e.g. using models to invert for source vertical mass distribution from satellite images and/or radar information).  Numerous techniques are possible, varying in hierarchy from users manually changing inputs, inverse modeling techniques (Stohl, 2011) or full variational data assimilation (as done by NWP models). Within dispersion modeling some of these techniques are currently immature and therefore the key issues here are to explore and evaluate potential techniques and also to determine the computational/timing implications for operational delivery.

Overall, there is a need for more validation of model forecasts against observational data (remote-sensing images and retrievals, airborne-sampling of the cloud, ground studies of ash deposits, etc.).  Validation is occurring now.  Volcanic Ash Advisory Center (VAAC) forecasters blend available volcanic ash observational data (usually in the form of satellite imagery) with dispersion model output to produce their volcanic ash advisory output although the weighting of the real-time volcanic ash observational data will decrease through the forecast period (18 hours for a standard VAA).  Some research groups are performing their own rigorous validation studies.  In addition to these individual efforts, a systematic and coordinated long-term validation program among many VAAC, their allied research units, and academic researcher promises to yield more improvements in dispersion modeling, to the benefit of both the scientific and user communities.

Implications for Operational Products

The objective of this overview is not to diminish the importance of dispersion modeling by pointing out the issue of forecast uncertainty, but rather to (1) demonstrate that the scientific community is well aware of and working on the issue and (2) give important context for the user community who must make decisions based on future conditions that cannot be fully known.

For operational users, the main conclusions are that the definition of the eruptive source term represents the largest source of error but that uncertainties in the modeling of meteorological and in particular dispersion processes also impact on predicted ash concentration values.  For the Eyja eruption of April/May 2010 the overwhelming majority of research aircraft ash concentration measurements taken in modeled ash concentration zones of between 0.2 to 20 mg/m3 were within one order of magnitude of the predicted peak concentration values.  However, this volcano was amongst the best monitored in the world and so the uncertainties associated with a different type of volcanic eruption in a remote area with an entirely different set of observational data and climatic conditions will inevitably be different.

This conclusion has significant implications for operational products, such as the EUR/NAT approach which uses specific ash-concentration values to outline areas of low, medium, and high ash contamination (see UK CAA guidance document at http://www.caa.co.uk/docs/1425/20110210GuidanceRegardingFlightOperationsInTheVicinityOfVolcanicAsh.pdf/).  In the EUR/NAT approach, an ash concentration value of 2 mg/m3 defines the boundary (contour line) between zones of medium and low ash contamination. This boundary, (assuming an order of magnitude error) could be recognized as representing a range from about 0.2 to 20 mg/m3 because of forecast uncertainties.  This means that the value of 4 mg/m3 used to define the boundary between medium and high contamination falls within the uncertainty range of the next lower boundary.  While this may seem odd it must be realized that the same uncertainty applies to both thresholds, so for 2ug/m3 the range might be 0.2 to 20, but for 4ug/m3 the range is 0.4 to 40 and at any point in this uncertainty range the “4” contour is still double the concentration of the “2” contour.  Therefore while overlap exists the 4ug/m3 contour still represents a higher level of risk and if used in this context such apparently small distinctions are perfectly valid.

At present, ash forecast products are deterministic in that they are derived from one dispersion model with one set of input data for eruption source parameters and forecast meteorology.  In Part II, we describe some options for probabilistic products that depict uncertainty in model output.

References Cited

Bonadonna, C., Folch, A., Loughlin, S., Puempel, H., : Ash Dispersal Forecast and Civil Aviation Workshop - Model Benchmark Document, https://vhub.org/resources/505, 2011.

Mastin, L.G., Guffanti, M., Servranckx, R., Webley, P., Bar- sotti, S., Dean, K., Durant, A., Ewert, J.W., Neri, A., Rose, W.I., Schneider, D., Siebert, L., Stunder, B., Swanson, G., Tupper, A., Volentik, A., Waythomas, C.F., A multi- disciplinary effort to assign realistic source parameters to models of volcanic ash-cloud transport and dispersion during eruptions, Journal of Volcanology and Geothermal Research (2009), doi: 10.1016/j.jvolgeores.2009.01.008.

Stohl, A., Prata, A. J., Eckhardt, S., Clarisse, L., Durant, A., Henne, S., Kristiansen, N. I., Minikin, A., Schumann, U., Seibert, P., Stebel, K., Thomas, H. E., Thorsteinsson, T., Tørseth, K., and Weinzierl, B.: Determination of time- and height-resolved volcanic ash emissions for quantitative ash dispersion modeling: the 2010 Eyjafjallajökull eruption, Atmos. Chem. Phys. Discuss., 11, 5541-5588, doi:10.5194/acpd-11-5541-2011, 2011.

Witham, C.S., Hort, M.C., Potts, R., Servranckx, R., Husson, P., and Bonnardot, F. 2007. Comparison of VAAC atmospheric dispersion models using the 1 November 2004 Grimsvotn eruption, Meteorological Applications, 14, 27-38.