Menzel / Purdom 

Costa Rica Remote Sensing Seminar

14-18 Mar 2005

Lab 1 - Staging, Viewing, Interrogating MODIS Data and Inferring Cloud Properties

Table: MODIS Channel Number, Wavelength ((m), and Primary Application

Reflective Bands





Emissive Bands

1,2
0.645, 0.865

land/cld boundaries
20-23
3.750(2), 3.959, 4.050
sfc/cld temp

3,4
0.470, 0.555

land/cld properties
24,25
4.465, 4.515

atm temperature

5-7
1.24, 1.64, 2.13

“


27,28
6.715, 7.325

water vapor

8-10
0.415, 0.443, 0.490
ocean color/chlorophyll
29
8.55


sfc/cld temp

11-13
0.531, 0.565, 0.653
“


30
9.73


ozone

14-16
0.681, 0.75, 0.865
“


31,32
11.03, 12.02

sfc/cld temp

17-19
0.905, 0.936, 0.940
atm water vapor

33-34
13.335, 13.635 

cld top properties 

26
1.375


cirrus clouds

35-36
13.935, 14.235

cld top properties

Table: Summary of Hydra Commands
HYDRA main window
Displays world map

Shows menus for Data, Settings, and Start at top.

Toolbar at bottom enables reset, zoom, translate, location, and subset functions 


Reset (toggle restores original image) 

Zoom in (left click plus drag enables enlargement)


Zoom out (left click plus drag enables far view)

Translate (left click plus drag enables moving the image within the window)


Rubber band zoom (left click plus drag to create box for enlargement)

Pick image (left click plus drag displays location of the chosen pixel)


Subset image (left click plus drag creates subset of image; this is automatically 

transferred from HYDRA into the Multichannel viewer when both are engaged)

Data menu

Local allows you to scroll through local directories to find MODIS, AIRS, or 

Meteosat data.
Remote allows you to find a MODIS or an AIRS granule from a remote server 

(a) Terra or Aqua Direct Broadcast or (b) Goddard DAAC.

After a data granule is selected HYDRA displays the infrared window on a world map.

    Exit closes this window
Settings menu
     Set color range allows you to adjust the color display (left click shows VisAD 

histogram – color range can be altered with a right click plus drag at either 

end of the color scale).

Set color scale gives choice of color, grey, or inverse grey (which produces white

 clouds).

Start opens Multichannel Viewer

wherein a spectra (wavenumber on x-axis and radiance on y-axis) is displayed along with a spectral band superimposed on a world map.  Left click on pick image in the bottom toolbar allows you to see the pixel value for a given lat-lon (using left click and drag).


Tools menu (appears in Multichannel Viewer window)
    Linear Combinations opens the channel combination tool display where you can 

specify linear combinations of spectral bands a,b,c and d 

(a   +-x /   b)   +-x /    (c   +-x /   d). 

Compute creates an image of the selected linear combination (you can indicate at 

the bottom your preference for this linear combination to be on the x- or y-

axis in the scatter plot) and displays a toolbar for image manipulations (five color boxes are available in the toolbar to select regions by boxes or curves in the image that will be displayed in the scatter plot; a left click drag in the image highlights the chosen points in the scatter plot).

Scatter  creates a scatter plot of the chosen x- and y-axis linear combinations. Five 

color area boxes (or area curves) in the bottom toolbar of the scatter plot 

allow you to select points in the scatter plot that will be displayed in the x-

axis and y-axis images; a left click drag in the scatter plot highlights the 

chosen points in the scatter plot and simultaneously in the x- and y-axis

images.  Conversely (as indicated before) left click drag in the x- or y- axis 

images shows the locations of the chosen pixels in the scatter plot.  Each 

color area box (or area curve) can be erased with a left click when the 

color box is engaged; after erasure another area box (or area curve) can be 

selected for this color.   The zoom capability in the scatter plot (on the 

bottom right of the toolbar) allows you to enlarge special features in the 

scatter plot and investigated them in more detail. 
    Transect allows you to create a line on the image and see the temperatures or 

radiances along the transect.  This is enabled with a left click drag. 

    Capture Display creates a jpeg of the image in a file location you specify.
    Statistics allows you to display min and max values and locations on image

 (toggle on or off).

Settings menu (appears in Multichannel Viewer window)
     Set color range opens VISAD histogram of brightness temperature (BT) values  

Radiance –> BT allows you to select radiance or brightness temperature in the display

Projection allows you to put the data in a given projection (mercator is the default)

Set Color Scale gives you the choice of color, grey, or inverted grey
Exercise 1 

1. Analyze the cloud scene over the Carribean on 6 Feb 2005 detected by MODIS using the Hydra software. Click on the menu data and then click on local. Go into the Data directory and select the file MOD021KM.A2005037.1845.004.2005039011528.hdf, click on the open button. Click on the start menu and then click on the Multi-Channel Viewer. Using the Multi-Channel Viewer window get familiar with the menu band, and menu Set Color Range and radiance->BT under the Settings tool.

2. Browse through the scene in several different wavelengths in the Multi-Channel Viewer window. Look at the scene both in radiances and BT (use radiance->BT under the Settings tool):

(a) verify that for window channels the surface features are visible while for absorption channels they are not;

(b) At  11 (m the BT ranges between  220 K and  320K . Can you locate where the minimum and maximum of BT for band 31 (11 (m) occur? 

(c) look at clouds, ocean, and vegetated surfaces in different reflective bands. For each type  (by selecting a single Field Of View) describe the changes in reflectance for different channels (top panel of Multi-channel viewer window).

3. Sub-select different regions (over clouds, land and ocean). In the Multi-channel viewer window click on the button with the array (sixth button from the left: pick image) and click on any pixel of your scene. The spectrum on the top panel of the Multi-Channel Viewer window will show the observed radiances for that channel: 

(a) describe for each scene the relative changes for the emissive bands at  11 (m;

(b) for each scene identify the most and the least reflective bands;

(c) for each scene try to draw by hand a curve for the reflectance as function of the wavelength;

4. Now select a region over the ocean and select band number 27 (6.7 (m) and display the image in BT. This band is used in cloud detection but also to derive upper tropospheric humidity (UTH). Describe most evident characteristics in the image.  Click on the menu command Tools and use transect. Approximately draw a line (point to the start point, use SHIFT+Right-Click+-Drag to the end point) as indicated below.  Repeat this for band number 31 (the IRW).  Note the differences in the transects for these two channels. 

[image: image1.jpg]



Band 27 Water vapor image

Exercise 2 

Select the following region (outlined approximately by green box) from the Hydra window. Now select band 4 (0.55 (m) and display in reflectance using the Settings menu (BT->radiance) button.

[image: image2.jpg]



Snow, Clouds, and Clear Sky

(a) Investigate the radiances emanating from the scene in different wavelengths; look at Bands 1 (0.65 (m), 2 (.86 (m), 6 (1.64 (m), 20 (3.7 (m), 31 (11 (m) and 35 (13.9 (m). Note how, in BT, clouds appear larger at 11 (m than at 3.7 (m (select those two bands and toggle back and forth between them). Can you explain why? 

(b) Comment on the cloud and clear sky characteristics in each of these spectral bands.  Which three bands would you choose to determine a cloud / no cloud "mask"; why? What radiative characteristics of the cloud, surface and atmosphere led you to select these spectral bands?  What reflectance or brightness temperature thresholds would you use? 

(c) Consider the Band 31 (11 um) image. What is the range of brightness temperatures over the land surface? What are the brightness temperatures of the different cloud types in the scene? Could you assume something about cloud phase from the cloud brightness temperatures?

(d) Use the Tools/Linear Combination math tool to combine spectral bands that can be used to detect clouds and infer cloud properties (hit return after entering numbers in the box). Try the following band combinations (by entering the suggested wavelengths in the colored boxes), and for each combination click on the compute button.

    i. Band 31 (11 (m)

    ii. Band 2 (0.86 (m ) / Band 1 (0.65 (m) 

    iii. (Band 2 (0.86 (m) - Band 1 (0.65 (m)) / (Band 2 (0.86 (m) + Band 1 (0.65 (m))

(e) Map one of the linear combinations to X axis (by clicking the X axis button) and another one to the Y axis (by clicking the Y axis button). Click on the scatter button to open a scatter plot window.

(f) Looking at the images of the linear combinations, estimate what threshold values you would use in each test to indicate the presence of clouds?  Select different regions in the scatter plot window and look at the corresponding points in the linear combination window (use the CTRL key and the right mouse button to select a rectangle or use the SHIFT key and the right mouse button to select a region by hand)

(g) Using the plot/Scatter-Plot math tool plot Band 31 (11(m) versus [Band 29 (8.6 (m) minus Band 31 (11 (m)]. Look at the image of [Band 29 (8.6 (m) minus Band 31 (11 (m)]. In the same way construct an image of [Band 31 (11 (m) minus Band 32 (12 (m)]. Note where the largest differences occur in each image. Describe what you see. Are there clouds in these brightness temperature difference images that could not be seen in the visible images? Close all the Scatter Plot figures.

Exercise 3

Now, in Hydra subselect the region shown below (use the Select button, the last on the right, in the Hydra window). Once the selected area shows up in the MultiChannelViewer window, select BT, and use the Tools/LinearCombination function

[image: image3.jpg]



Clouds and clear sky
(a) Make an image of Band 31. To do so de-select the minus sign between the red and green boxes on the Channel Combination Tool and then click the Compute button. Then make an image of Band 31 - Band 32. Map Band 31 on the x-axis (click on the button X-Axis) and the difference between Band 31 and Band 32 on the y-axis (click on the button Y-Axis) and click the Scatter button. 

(b) Map Band 31 on the x-axis and Band 31 – Band 32 on the y-axis (by clicking on the circular buttons in the pseudo-channel windows) and click on the Scatter button. Now you can select a rectangular region on the scatter plot and look at the images to determine the location of the selected points in the pseudo-images. Alternatively use the system to select a rectangular region of one of the pseudo-images that display the linear combinations and look at the position of the selected pixels in the scatter plot. Try to determine the type of cloud viewed in this region. Did you notice these clouds in the band 4 image earlier in the lab? Does Band 26 (1.38 µm) support or contradict your cloud classification. If you were to design and automated algorithm to detect these cloud types, what would you include in your algorithm?
(c ) Make a new linear combination Band 29 (8.6  (m) minus Band 31 (11 (m). Look at the range of point values that occur along each axis. Which difference is larger for this cloud, [Band 29 (8.6  (m) minus Band 31 (11 (m)] or [Band 31 (11 (m) minus band 32 (12 (m)]? 










